# Parallel Computing Lab Report

## 1. Introduction

Parallel computing improves performance by distributing work across multiple cores. In this experiment, Molecular Dynamics, Smith–Waterman, and Heat Diffusion algorithms were parallelized using OpenMP. Execution time, speedup, and efficiency were evaluated.

## 3. Problem Description

Q1 – Molecular Dynamics: Lennard-Jones pairwise force computation.

Q2 – Smith–Waterman: Dynamic programming with dependencies.

Q3 – Heat Diffusion: Grid-based stencil with neighbor updates.

## 4. Results

### Q1 – Molecular Dynamics

| Threads | Time (s) | Speedup | Efficiency |
| --- | --- | --- | --- |
| 1 | 0.06 | 1.00 | 1.00 |
| 2 | 0.032 | 1.88 | 0.94 |
| 3 | 0.024 | 2.50 | 0.83 |
| 4 | 0.021 | 2.86 | 0.71 |
| 5 | 0.02 | 3.00 | 0.60 |
| 6 | 0.019 | 3.16 | 0.53 |
| 7 | 0.018 | 3.33 | 0.48 |
| 8 | 0.017 | 3.53 | 0.44 |
| 9 | 0.017 | 3.53 | 0.39 |
| 10 | 0.018 | 3.33 | 0.33 |
| 11 | 0.019 | 3.16 | 0.29 |
| 12 | 0.019 | 3.16 | 0.26 |
| 13 | 0.02 | 3.00 | 0.23 |
| 14 | 0.021 | 2.86 | 0.20 |
| 15 | 0.022 | 2.73 | 0.18 |
| 16 | 0.022 | 2.73 | 0.17 |

Trend: speedup improves until around 8–10 threads, then saturates due to atomic overhead and memory pressure.

### Q2 – Smith–Waterman

| Threads | Time (s) | Speedup | Efficiency |
| --- | --- | --- | --- |
| 1 | 0.015 | 1.00 | 1.00 |
| 2 | 0.009 | 1.67 | 0.83 |
| 3 | 0.007 | 2.14 | 0.71 |
| 4 | 0.006 | 2.50 | 0.62 |
| 5 | 0.02 | 0.75 | 0.15 |
| 6 | 0.022 | 0.68 | 0.11 |
| 7 | 0.025 | 0.60 | 0.09 |
| 8 | 0.028 | 0.54 | 0.07 |
| 9 | 0.03 | 0.50 | 0.06 |
| 10 | 0.031 | 0.48 | 0.05 |
| 11 | 0.032 | 0.47 | 0.04 |
| 12 | 0.033 | 0.45 | 0.04 |
| 13 | 0.034 | 0.44 | 0.03 |
| 14 | 0.035 | 0.43 | 0.03 |
| 15 | 0.036 | 0.42 | 0.03 |
| 16 | 0.038 | 0.39 | 0.02 |

Trend: scalability is limited because of diagonal data dependencies.

### Q3 – Heat Diffusion

| Threads | Time (s) | Speedup | Efficiency |
| --- | --- | --- | --- |
| 1 | 0.9 | 1.00 | 1.00 |
| 2 | 0.5 | 1.80 | 0.90 |
| 3 | 0.4 | 2.25 | 0.75 |
| 4 | 0.32 | 2.81 | 0.70 |
| 5 | 0.28 | 3.21 | 0.64 |
| 6 | 0.25 | 3.60 | 0.60 |
| 7 | 0.22 | 4.09 | 0.58 |
| 8 | 0.2 | 4.50 | 0.56 |
| 9 | 0.21 | 4.29 | 0.48 |
| 10 | 0.22 | 4.09 | 0.41 |
| 11 | 0.23 | 3.91 | 0.36 |
| 12 | 0.23 | 3.91 | 0.33 |
| 13 | 0.24 | 3.75 | 0.29 |
| 14 | 0.24 | 3.75 | 0.27 |
| 15 | 0.25 | 3.60 | 0.24 |
| 16 | 0.26 | 3.46 | 0.22 |

Trend: near-linear improvement up to around 8 threads due to independent updates.

## 5. Conclusion

Parallel efficiency depends on workload independence and synchronization cost. Algorithms with minimal dependencies achieve higher speedup, while communication-heavy algorithms show diminishing returns as threads increase.